// SRAM 8x8

`timescale 1ns / 1ps

module sram_8x8(

input  wire       clk,

input  wire [7:0] data_in,

input  wire       we,        // 1 = write, 0 = read

input  wire [2:0] address,

output reg  [7:0] data_out

);

reg [8:0] mem [8:0]; // 8x8 storage

integer i;

// Initialize memory

initial begin

for (i = 0; i < 8; i = i + 1)

mem[i] = 8'h00;

data_out = 8'h00;

end

// Write / read logic

always @(posedge clk) begin

if (we) begin

mem[address] <= data_in;   // Write

data_out <= data_in;       // Write-through (see new data immediately)

end else begin

data_out <= mem[address];  // Read

end

end

endmodule

// Sensor Interface

`timescale 1ns / 1ps

module sensor_interface (

input  wire [7:0] sensor_data,

input  wire       valid,

input  wire       clk,

output reg        trigger_write,

output reg [7:0]  data_out

);

always @(posedge clk) begin

if (valid) begin

trigger_write <= 1'b1;

data_out <= sensor_data;

end else begin

trigger_write <= 1'b0;

end

end

endmodule

// top module

`timescale 1ns / 1ps

module baby_monitor_top (

input  wire       clk,

input  wire       reset,

// Sensor Inputs

input  wire [7:0] heartbeat_data,

input  wire       heartbeat_valid,

input  wire [7:0] temperature_data,

input  wire       temperature_valid,

input  wire [7:0] motion_data,

input  wire       motion_valid,

// Outputs

output wire [7:0] alert_data,

output wire       heartbeat_alert,

output wire       temperature_alert,

output wire       motion_alert

);

// Internal wires

wire we_hb, we_temp, we_motion;

wire [7:0] hb_out, temp_out, motion_out;

wire [7:0] sram_out;

reg  [2:0] addr;

// ---------------------------------------------------

// Sensor Interfaces

// ---------------------------------------------------

sensor_interface hb_sensor (

.sensor_data(heartbeat_data),

.valid(heartbeat_valid),

.clk(clk),

.trigger_write(we_hb),

.data_out(hb_out)

);

sensor_interface temp_sensor (

.sensor_data(temperature_data),

.valid(temperature_valid),

.clk(clk),

.trigger_write(we_temp),

.data_out(temp_out)

);

sensor_interface motion_sensor (

.sensor_data(motion_data),

.valid(motion_valid),

.clk(clk),

.trigger_write(we_motion),

.data_out(motion_out)

);

// ---------------------------------------------------

// SRAM (shared buffer for all sensors)

// ---------------------------------------------------

wire       we_any;

wire [8:0] data_to_write;

assign we_any = we_hb | we_temp | we_motion;

// Priority: heartbeat > temperature > motion

assign data_to_write = we_hb ? hb_out :

we_temp ? temp_out :

motion_out;

sram_8x8 sram (

.clk(clk),

.data_in(data_to_write),

.we(we_any),

.address(addr),

.data_out(sram_out)

);

// ---------------------------------------------------

// Address counter (circular buffer)

// ---------------------------------------------------

always @(posedge clk or posedge reset) begin

if (reset)

addr <= 0;

else if (we_any)

addr <= addr + 1;

end

// ---------------------------------------------------

// Threshold alert logic

// ---------------------------------------------------

parameter HB_THRESHOLD   = 8'd120;

parameter TEMP_THRESHOLD = 8'd100;

parameter MOTION_THRESHOLD = 8'd1;

reg hb_alert_r, temp_alert_r, motion_alert_r;

always @(posedge clk or posedge reset) begin

if (reset) begin

hb_alert_r    <= 0;

temp_alert_r  <= 0;

motion_alert_r<= 0;

end else begin

if (heartbeat_valid && heartbeat_data > HB_THRESHOLD)

hb_alert_r <= 1;

else

hb_alert_r <= 0;

if (temperature_valid && temperature_data > TEMP_THRESHOLD)

temp_alert_r <= 1;

else

temp_alert_r <= 0;

if (motion_valid && motion_data >= MOTION_THRESHOLD)

motion_alert_r <= 1;

else

motion_alert_r <= 0;

end

end

assign heartbeat_alert   = hb_alert_r;

assign temperature_alert = temp_alert_r;

assign motion_alert      = motion_alert_r;

// ---------------------------------------------------

// Output Data (latest stored value)

// ---------------------------------------------------

assign alert_data = sram_out;

endmodule

// Test bench

`timescale 1ns / 1ps

module simulation;

// Testbench signals

reg clk, reset;

reg [7:0] heartbeat_data, temperature_data, motion_data;

reg heartbeat_valid, temperature_valid, motion_valid;

wire [7:0] alert_data;

wire heartbeat_alert, temperature_alert, motion_alert;

// Instantiate DUT

baby_monitor_top uut (

.clk(clk),

.reset(reset),

.heartbeat_data(heartbeat_data),

.heartbeat_valid(heartbeat_valid),

.temperature_data(temperature_data),

.temperature_valid(temperature_valid),

.motion_data(motion_data),

.motion_valid(motion_valid),

.alert_data(alert_data),

.heartbeat_alert(heartbeat_alert),

.temperature_alert(temperature_alert),

.motion_alert(motion_alert)

);

// Clock generation (10ns period)

always #5 clk = ~clk;

// Stimulus

initial begin

// Init

clk = 0;

reset = 1;

heartbeat_data = 0;

temperature_data = 0;

motion_data = 0;

heartbeat_valid = 0;

temperature_valid = 0;

motion_valid = 0;

// Release reset

#15 reset = 0;

// Send heartbeat value = 80 (normal)

#10 heartbeat_data = 8'd80; heartbeat_valid = 1;

#10 heartbeat_valid = 0;

// Send heartbeat value = 130 (abnormal, should trigger alert)

#20 heartbeat_data = 8'd130; heartbeat_valid = 1;

#10 heartbeat_valid = 0;

// Send temperature value = 90 (normal)

#20 temperature_data = 8'd90; temperature_valid = 1;

#10 temperature_valid = 0;

// Send temperature value = 110 (abnormal, should trigger alert)

#20 temperature_data = 8'd110; temperature_valid = 1;

#10 temperature_valid = 0;

// Send motion = 0 (no movement)

#20 motion_data = 8'd0; motion_valid = 1;

#10 motion_valid = 0;

// Send motion = 1 (movement detected)

#20 motion_data = 8'd1; motion_valid = 1;

#10 motion_valid = 0;

// Let simulation run a little

#50 $finish;

end

// Monitor values

initial begin

// Monitor values

$monitor("Time=%0t | HB=%d (alert=%b) | TEMP=%d (alert=%b) | MOTION=%d (alert=%b) | alert_data=%d",

$time, heartbeat_data, heartbeat_alert,

temperature_data, temperature_alert,

motion_data, motion_alert,

alert_data);

end

endmodule